School of Engineering

- 1E10 Assignment.

- A2. Dynamics for the Mangonel-With Drag

Name: Kris Vanhoutte

Student No. 12301975

- 1E10 Assignment.

- A2. Dynamics for the Mangonel-With Drag

Complete the following tasks and submit using WebCT.

- Save this word document adding your name and student number to the front page. Using the Excel spreadsheets that you have developed to model the dynamics of a “missile” cast by the mangonel which is subject to aero-dynamic drag, cut and paste charts for the following  parameters into the document

  - Drag Vs No Drag. Velocity=30m/s, Angle=70 degrees

  - Drag Vs No Drag. Velocity=30m/s, Angle=20 degrees

The values rho=1.2 kg/m^3, Cd=0.4, mass=0.05kg, D=0.045 m are to be used in this question.

See note 1 below regarding graph presentation.

- Read, from the figures determined in Q1 or directly from the results calculated using your spreadsheets, the predicted distances travelled in the x-direction for the “drag” and “no-drag” cases. Complete the following table. Round your results to nearest integer (no decimal places).

|  | 70 Degrees | 20 Degrees |
| --- | --- | --- |
| x (with drag) [m] | 38.35 m | 45.51 m |
| x (no drag) [m] | 58.90 m | 58.92 m |

- From the results of question 2, write one or two sentences to address each of the following; [See end of document]

    - observations,

    - explanations and

    - recommendations with regards to the optimum use of the Mangonel.

- If we were to test the mangonel outside in windy conditions, what comments have you to add based on the analysis above. [See end of document]

- Complete the following table for the maximum distance travelled, x.

| Cd \ Velocity | 10m/s | 20m/s | 40m/s |
| --- | --- | --- | --- |
| 0 | 10.18 m | 40.73 m | 162.92 m |
| 0.5 | 9.46 m | 31.52 m | 79.65 m |
| 1.0 | 8.83 m | 26.15 m | 55.49 m |

The values rho=1.2 kg/m^3, mass=0.05kg, D=0.045m, theta=45degrees are to be used in this question.

- From the results of question 5, write one or two sentences to address each of the following; [See end of document]

    - observations,

    - explanations and

    - recommendations with regards to the optimum use of the Mangonel.

- Complete the following table for the maximum distance travelled, x.

| mass \ Velocity | 10m/s | 20m/s | 40m/s |
| --- | --- | --- | --- |
| 0.010 [kg] | 7.85 m | 19.84 m | 35.89 m |
| 0.050 [kg] | 9.56 m | 32.98 m | 87.86 m |
| 0.100 [kg] | 9.89 m | 36.37 m | 112.27 m |

The values rho=1.2 kg/m^3, Cd=0.4, D=0.045m, Theta=45degrees are to be used in this question.

- From the results of question 7, write one or two sentences to address each of the following;  [See end of document]

    - observations,

    - explanations and

    - recommendations with regards to the optimum use of the Mangonel. For this last point, consider the effect of a change in mass when all other aerodynamic and physical parameters remain fixed, e.g. the potential energy stored in the “spring”.

- Submit your excel spread sheet as an attachment, with a chart plotted for the 20m/s, 45 Degree case.

The values rho=1.2 kg/m^3, Cd=0.4, mass=0.05kg, D=0.045 m are to be used in this question.

Notes:

- Ensure the curves are visible and sufficient resolution is provided so that the height and distance is determinable. The following chart is an example for 80 degrees at 30m/s

- MS Word document and Excel spreadsheet to be created so that they can be opened with Office 2003 or Office 2007. If your documents cannot be opened they will receive no marks.

- Submit your documents on time. No extensions ext. will be granted.

- Submit your documents by uploading to WebCT only. Do not attach to email etc.

- Ensure you can logon and that all such issues are resolved in good time before the deadline. Passwords expire. Do not leave this assignment until the last minute to find you have some IT issue.

- Cheating will result in zero marks for the assignment and possibly for the year. Your graphs should be linked to your Excel spreadsheet results. Manually creating curves in Excel is considered cheating. Those students who have done so have been noted.

Despite this list, try and enjoy the assignment and try to think around the subject as much as possible and take from it any tips that you might use with your own mangonel.

Marking Scheme:

- 2 per figure [max. 4]

- 1 per result. (within 4 integers) [max. 4]

- 4 per point included [max. 20]

- 4 for a good point [max. 4]

- 1 per result [max. 9]

- 4 per point included [max. 24]

- 1 per result [max. 9]

- 4 per point included [max. 24]

- 4 for correct Excel sheet, 4 for figure. [max. 8]

Total = 106

Scale to 100, i.e. (x/106)*100

Question 3.

I. It can be seen from these results that the angle of projection is symmetric about 45 degrees for the mangonel without drag, and the distance travelled is almost exactly the same. Whereas with drag, an angle which is lower than 45 degrees results in a further distance than one which is greater than 45 degrees.

II. The reason that a lower angle results in further distance travelled is because as the angle of projection increases, the initial velocity in the x direction, i.e. the horizontal direction also increases. As well as this, because the ball has a smaller velocity in the y direction, it will stay in the air for a shorter time, and so the effect of drag in the horizontal direction will be less than if it was to stay in the air for a longer time.

III. The best angle of projection is still 45 degrees, but if going for a different angle, an angle less than 45 degrees will reach a further distance than one above.

Question 4.

Based on the analysis above, I would say that testing the mangonel in windy conditions would have a huge effect on the distance travelled by the ball. Depending on the direction of the wind relevant to the line of fire, this could hugely increase the drag, in which case the ball would travel a shorter distance, or if the mangonel is facing in the opposite direction, the addition of the wind could increase the distance travelled by the ball, relative to conditions without any wind.

Question 6.

I. These results show that a lower drag coefficient increases the distance travelled for all speeds, with an optimal drag coefficient being 0, i.e. no drag at all.

II. The lower the drag coefficient, the closer to a situation with no drag (i.e. an ideal situation) our projectile is. Furthest distance is achieved with no drag present and so it is clear why a lower drag increases distance travelled.

III. Keeping the drag coefficient as low as possible will allow the furthest distance to be achieved. This can be achieved by using a projectile which has a small drag coefficient, in this case a small ball. Ideally some sort of streamlined body, similar to an airfoil, would make the optimum projectile as its drag coefficient is around one tenth of that of a sphere, ~0.04 for an airfoil vs. ~0.4 for a sphere.

Question 8.

I. It’s clear from the results that an increase in mass corresponds with an increase in distance travelled for all speeds. Although as the mass increases, the distance travelled does not increase at an equal rate.

II. The mass is inversely proportional to the constant K, and so an increase in mass reduces the acceleration due to drag (i.e. a negative acceleration effecting the projectile). This acceleration can only become 0 though, as increasing the mass does not cause a force to act in the opposite direction. Because of this it can be seen that as the mass approaches higher values, it has smaller and smaller effects on the acceleration.

III. There is clearly an ideal mass for the ball, which reduces the negative acceleration past a certain point. From our data, any value of mass between ~0.1kg and ~0.3kg will reduce the negative acceleration enough to bring the distance travelled close to that in a case where there is no drag, without significantly increasing the force required to launch the projectile.